BUBU- Toy Pusher Tidy Bot

A small robot with a front scoop drives around the floor, finds small objects such as toys, and pushes them to a corner or into a box. It doesn't pick anything up, so the build stays simple. It sorts "object" from "wall" using ultrasonic distance patterns.

The parts

- Body: a small car chassis with two wheels driven by two motors (steered by running one wheel faster than the other), plus a caster wheel for balance.

- Scoop plate: a flat or curved plate on the front, like a bulldozer blade. It keeps the toy in front of the robot instead of letting it slide off sideways.

- Eyes: ultrasonic sensors that measure distance by sending a sound pulse and timing the echo.

- Brain: an Arduino or ESP32 running the logic.

How it works, step by step

- Search: the robot drives around the room slowly and keeps checking distances.

- Decide what it sees: this is the clever part. A wall and a toy both give a "something is close" reading, so you need a trick to tell them apart. The simplest one is to mount two sensors at different heights. The low sensor (about 3 cm off the floor) sees both toys and walls. The high sensor (about 15 cm up) sees only walls, because toys are short. If the low sensor says "close" and the high one says "far," it's a toy. If both say "close," it's a wall.

- Push: once it spots a toy, it drives straight at it with the scoop and pushes it forward.

- Walk it to the corner: it pushes the toy until it reaches a wall, then follows the wall (staying a fixed distance from it) until the front and side sensors both say "close." That means it has reached a corner.

- Release and repeat: it backs up, leaving the toy in the corner, turns around, and goes back to searching.

Limits to know about

- Soft toys like plushies absorb sound and can be hard for the sensor to see.

- Ultrasonic sensors have a blind spot of about 2 cm right in front of them, and very small toys can be missed.

- Start in a simple arena, such as a box-shaped area with a few wooden blocks, before trying a real room.

Build in three levels

- Level 1: drive forward and turn away from obstacles (basic obstacle avoidance).

- Level 2: add the two-height sensing so it can tell toy from wall.

- Level 3: add the push-to-corner routine and wall following.

1. Tape Zones (virtual rules for the robot)
Two cheap IR line sensors on the underside let the robot read tape on the floor. Black tape means "never enter," so a child can circle their Lego build and the bot will leave it alone. Green tape means "drop zone," so toys go there instead of any corner. Rules are set with tape, with no app and no mapping.
Cost: about ₹100 to 150 for sensors, plus tape.

2. "I Need Help" Messages
A current sensor on the motor (or just the ESP32 reading motor load) notices when the robot is pushing something too heavy or is stuck. It stops, remembers the spot, and sends a message to your phone like "Stuck at the sofa side, something heavy." It marks that item as "skip" so it doesn't try again. Most robots just keep struggling, so a bot that knows its limits and asks for help is a nice touch.
Cost: about ₹150 for the current sensor (or free using the motor driver's load readings).

3. Whisker Bumper for Soft Toys
Ultrasonic sensors can't see plushies because soft surfaces absorb sound. Add a front bar with two tiny microswitches (a bumper or whiskers). The software combines the two senses: distance says "something might be there," and the bumper confirms "yes, it's soft and real." This fixes the project's biggest weakness and is a good point for your report.
Cost: about ₹50 to 100.

4. Tidy Race Mode (a game for kids)
Kids race the robot. They start a timer from the phone page, grab toys by hand, and the robot pushes toys at the same time. The page shows who cleared more in two minutes and keeps a high-score list. It turns cleaning into a game and makes a memorable demo.
Cost: free, since it's just software on the ESP32's built-in web page.

6. Clap-to-Start and Tap-Teach Home

- A cheap sound module lets you clap twice to start the robot.

- To teach it where home is, push it to your chosen corner and press one button. It remembers the spot by counting wheel turns, so you don't need to install any beacon.
Cost: about ₹50 for the sound module, plus a few rupees for wheel encoders (or use the tape zone from #1 instead).

7. Missing Toy Alert
The robot counts how many toys it delivers to the drop zone each session. Over a few days it learns your usual number (for example, about 12). If today's count comes in much lower, it tells your phone: "Only 8 toys found. 4 may be hiding under furniture." So the robot reports on what it couldn't reach, not just what it cleared.
Cost: free, software only. It counts when the whisker bumper clicks at the drop zone.

8. Under-Furniture Drag Hook
Make the robot's body low (under 8 cm tall) and add a flat hook plate on its back. It drives under a bed or sofa, then reverses and drags toys out into the open, where the front scoop can push them to the drop zone. Most tidy robots only push forward, so the two-way routine is the interesting part.
Cost: about ₹20, a piece of cardboard or a 3D-printed plate.

9. Pause When Humans Are Near
A small PIR motion sensor tells the robot when a person or pet walks close. It stops, plays a soft beep, and waits a few seconds before starting again. This protects toes and pets, which matters for a robot meant to be used around children.
Cost: about ₹60.

Hardware list

| Part | Qty | Used for | Est. cost (₹) |
| --- | --- | --- | --- |
| ESP32 dev board | 1 | Brain, Wi-Fi, phone web page | 350-450 |
| 2WD chassis kit (2 gear motors, wheels, caster) | 1 | Body and driving | 280-350 |
| L298N motor driver | 1 | Controls the motors | 100 |
| HC-SR04 ultrasonic sensor | 2 | Toy vs wall detection (low and high) | 140 |
| TCRT5000 IR line sensor | 2 | Tape Zones (black tape = no-go) | 80 |
| Lever microswitch | 2 | Whisker Bumper, toy counting | 20-30 |
| Sound sensor module | 1 | Clap-to-Start | 50 |
| PIR motion sensor | 1 | Pause When Humans Are Near | 60 |
| Buzzer and push button | 1 each | Alerts and manual start | 20 |
| 4xAA holder plus 4 cells | 1 | Motor power | 100 |
| Old phone power bank (if you have one) | 1 | Power for the ESP32 | 0 |
| Cardboard or ice-cream-stick scoop and hook | - | Front scoop and Drag Hook | 20 |
| Jumper wires, small breadboard, 2 resistors | - | Connections and the echo-pin voltage divider | 130 |
| Black tape, double-sided tape, zip ties | - | Zone tape and mounting | 50 |

What changed to save money

- Color sensor and wheel encoders removed (about ₹330 to 470 saved). The drop zone is now simply the first corner the robot finds by wall following, and black tape is the only tape rule. Tap-Teach Home is dropped because it needed the encoders.

- Current sensor removed. The "I Need Help" feature now works by checking whether the front distance stops getting smaller while the robot is pushing. If nothing changes for about 3 seconds, it assumes the toy is stuck or too heavy, stops, and alerts you. It costs nothing and works well enough.

- Cheaper power. Use a power bank you already own for the ESP32 and 4 AA cells for the motors. Connect the grounds of both together. If you have an old laptop battery, you can also salvage 18650 cells for free.

- Software features cost nothing: Tidy Race Mode, Missing Toy Alert, and the phone alerts all run on the ESP32's own web page. Telegram alerts are also free if you have home Wi-Fi.

Even cheaper (about ₹1,200)
Drop the PIR, sound module, and IR sensors. You keep the core robot plus the Whisker Bumper, Drag Hook, "I Need Help," Race Mode, and Missing Toy Alert. You can add the rest later because each one is a small plug-in part.

Features kept: Tape Zones (black only), "I Need Help," Whisker Bumper, Tidy Race Mode, Clap-to-Start, Missing Toy Alert, Under-Furniture Drag Hook, Pause When Humans Are Near.

Make Bubu look like a character

- Googly eyes: stick two big ones on the front, just above the scoop. It instantly looks alive.

- Cute cover: wrap the body in colored paper, craft foam, or felt. Add rosy cheeks and a small smile with a marker.

- Tiny accessories: a bow, a small hat, or a paper cape (kids love "Super Bubu").

- Name badge: a little tag on the robot saying "Hi, I'm Bubu!"

Make Bubu feel alive
5. Happy beeps: a short "boop-boop" when it starts and a little happy tune when it delivers a toy.
6. Sad beeps: a low "ohh" tone when it gets stuck. With the "I Need Help" feature, kids will rush to help it.
7. LED eyes (optional, about ₹20): two small LEDs that blink, or turn green when happy and red when stuck.

Build a mini world for Bubu
8. Bubu's playground: a floor area about 1.5 m x 1.5 m, bordered with cardboard walls or books, so the robot has a clear arena.
9. Toy box with a name: label the drop zone "Bubu's Toy House" and decorate it like a little house.
10. No-go zone with a sign: circle a Lego build with black tape and add a sign, "Bubu says: please don't touch my friends' castle!"
11. Scattered toys: use soft balls, small blocks, and light toys, and start the demo with them scattered everywhere.

Make visitors join in
12. "Beat Bubu" challenge: kids race Bubu to clear toys in 2 minutes. Keep a score board with names.
13. Sticker reward: give a "I raced Bubu!" sticker to everyone who plays.
14. Clap to wake Bubu: invite kids to clap twice. It makes them part of the show.
15. Daily star: write the best score of the day on a big board.

Poster and banner tips
16. Big title: "Meet Bubu, the toy-tidying robot!"
17. A tagline in simple words: "Bubu cleans up, so you can play more."
18. A 3-step picture strip: Bubu finds the toy → pushes it → drops it in the toy house. Use big drawings, not text.
19. A small "How Bubu works" box: just two lines about the sensors, for adults who ask.

Quick checklist for the day

- Fully charge the batteries and keep spare AA cells.

- Test Bubu on the table at the venue, since different floors can behave differently.

- Keep a small tool kit (screwdriver, tape, extra wires).

- Keep a few toys that Bubu handles well, and avoid very heavy or very soft ones during the demo.